| # SPLIT RAIL FENCE INSTALLATION |  |  |
| --- | --- | --- |
| West Virginia Split Rail proves itself in ease of installation. There is no butting of rails into posts, eliminating exact post setting. The approximate 6” lap on each end of our West Virginia Split Rail will allow for irregularities in post setting without detracting from the overall finished appearance. |  |  |

# Getting Started

| ☐ | Using a string line, lay out the corners and ends of the fence. |
| --- | --- |
| ☐ | Dig first hole at starting corner or end, set post, and backfill for stability. |
| ☐ | Dig next hole about 10’6” from starting corner or end, install post, and partially backfill. Note: 10’6” distance is approximate. DO NOT dig all post holes in advance as they may vary slightly. |
| ☐ | Insert rails and finish backfilling. |
| ☐ | Follow steps 3 and 4 until you have finished installing your fence. |

Notes:

- If the distance that the fence covers cannot be worked in multiples of 10’6”, then the distance between the last two posts will be less than rail length and the rails must be cut off. Allow enough length to put a new scarf on the rail by using a hand saw or power saw.

- To compensate for short rails, you may want to divide the shortage by reducing the length of rails on both the first and last sections. If this is not required, then it is recommended that the short section be in a less conspicuous place.

# Additional Information for Setting Posts

| ☐ | - Dig hole 24” to 30” deep. Exact depth will be dependent on how high you want the first rail above the ground. |
| --- | --- |
| ☐ | Place post in the hole, add 12” of dirt, compact, and repeat. Do not finish setting post until next section of post, plus rail, is installed. |
| ☐ | Compact the backfill by tamping and sloping it away from the post for drainage. |
| ☐ | Optional – If you want to use concrete, place the post in the hole, fill with 12” of dirt, and compact. Fill the balance of the hole with concrete. This will form a concrete collar and will allow moisture to drain away from the bottom of the post. The drainage is critical for the longevity of the post. |